# Industrial Pump Vendor Inspection Report

SYNTHETIC DEMO DATA — NOT A REAL INSPECTION OR ENGINEERING DECISION

Purpose. This fictional report is designed only for Inspectra demonstrations. It contains multiple explicit findings so extraction, classification, review priority, and evidence highlighting can be demonstrated safely.

# Inspection summary

| Field | Value |
| --- | --- |
| Report ID | SYN-GEN-2026-021 |
| Vendor | Demo Process Equipment Services |
| Customer asset | Cooling-water pump P-204 |
| Equipment | Fictional centrifugal pump and motor skid |
| Inspection date | 10 September 2026 |
| Overall disposition | Repair before extended operation |

# Scope and method

Visual condition review, vibration screening, seal check, guarding inspection, instrument status review, and electrical enclosure examination. This document is generic synthetic training material for defect extraction.

# Findings

| ID | Component or location | Observed condition | Priority | Recommended action |
| --- | --- | --- | --- | --- |
| GP-01 | Drive-end bearing | Overall vibration measured 8.6 mm/s RMS; audible bearing noise present. | High | Inspect bearing and alignment; replace bearing if confirmed degraded. |
| GP-02 | Mechanical seal | Continuous water drip of approximately 12 drops per minute at seal housing. | High | Inspect seal faces and flush connection; repair leakage. |
| GP-03 | Coupling guard | Lower guard fixing bolt is missing; guard moves when touched. | High | Install compatible bolt and verify guard security. |
| GP-04 | Pressure gauge PG-204 | Calibration label expired and gauge pointer does not return to zero. | Medium | Replace or calibrate gauge before using it for decisions. |
| GP-05 | Motor terminal box | Cable gland is loose and unused entry is not blanked. | Medium | Secure gland and install approved blanking plug. |

# Inspector narrative

The drive-end bearing produced audible noise and vibration measured 8.6 mm/s RMS. A continuous drip was observed at the mechanical seal. The coupling guard had a missing lower fixing bolt and moved under light hand pressure.

The pressure gauge calibration label had expired and the pointer did not return to zero. A loose cable gland and an unblanked enclosure entry were present at the motor terminal box.

# Disposition

Place the pump in the corrective-maintenance queue. Correct the guard, leakage, bearing, gauge, and enclosure findings before extended operation. This synthetic report does not replace site safety procedures.

# Corrective action closeout

| Finding | Assigned role | Target date | Closeout status |
| --- | --- | --- | --- |
| GP-01 | Maintenance owner | 17 September 2026 | Open — verification required |
| GP-02 | Maintenance owner | 17 September 2026 | Open — verification required |
| GP-03 | Maintenance owner | 17 September 2026 | Open — verification required |
| GP-04 | Maintenance owner | 17 September 2026 | Open — verification required |
| GP-05 | Maintenance owner | 17 September 2026 | Open — verification required |

Inspector declaration. All information in this document is fictional and intended exclusively for software demonstration and testing.